STATE WATER RESOURCES CONTROL BOARD

ORDER NO. 2005-?

STATEWIDE GENERAL WASTE DISCHARGE REQUIREMENTS

FOR

SEWAGE COLLECTION SYSTEM AGENCIES

The State Water Resources Control Board, (hereinafter Water Board), finds that:

- All Federal and State agencies, municipalities, counties, districts, or other public bodies, which own or have responsibilities for sanitary sewer systems or any facilities that collect or convey untreated sewage in the State of California, are required to be named as Permittees in this Order.  (Note - I will reference a list of Agencies that are considered Permittees within this Order)

- Sanitary sewer overflows (SSOs) consist of varying mixtures of domestic sewage, and industrial and commercial wastewater depending on the pattern of land uses in the sewage collection system tributary area.  SSOs often contain high levels of suspended solids, pathogenic organisms, toxic pollutants, nutrients, oxygen demanding organic compounds, oil and grease and other pollutants.  SSOs may cause a nuisance, cause a temporary exceedance of applicable water quality standards when the sewage is discharged to surface waters of the State, pose a threat to the public health, adversely affect aquatic life, and impair the public recreational use and aesthetic enjoyment of surface waters.  Section 13050(m) of the California Water Code defines a nuisance as anything that meets the following requirements: (1) Is injurious to health, or is indecent to the senses, or an obstruction to the free use of property, so as to interfere with the comfortable enjoyment of life or property. (2) Affects at the same time an entire community or neighborhood, or any considerable number of persons, although the extent of the annoyance or damage inflicted upon individuals may be unequal. (3) Occurs during, or as the result of, the treatment or disposal of waste.

- SSOs occur frequently throughout the State.  While the causes of SSOs are specific to each system, some of the chief causes of SSOs include: grease blockages, root blockages, sewer line flood damage, manhole structure failures, vandalism, pump station mechanical failures, power outages, storm or ground water inflow/infiltration, debris blockages, sanitary sewer system age and construction material failures, lack of proper operation and maintenance, lack of capacity and contractor caused damages.   Many of these SSOs are preventable with adequate and appropriate source control measures and operation and maintenance of the sanitary sewer system.

- A reduction of SSOs will decrease risk to both human health and the environment;

- The number and volume of SSOs can be reduced through the application of sound and appropriate operation, infrastructure maintenance, and management principles;

- To facilitate proper funding and management of sanitary sewer systems, each sanitary sewer system owner or operator, including state owned facilities, should develop and implement a system-specific Sewer System Management Plan (SSMP).  To be effective, SSMPs should include provisions to provide proper and cost effective management, operation, and maintenance of sanitary sewer systems, while taking into consideration risk management and cost benefit analysis;

- Many local public agencies in California have already developed SSMPs and implemented measures to reduce SSOs, but others still require technical assistance and, in some cases, funding to accomplish this goal;

- The development of standards or bench-marks for collection system capital improvement, maintenance, operation, and performance will inform compliance with these requirements.  Furthermore, SSMP certification to these standards by technically qualified and experienced persons, will provide a useful and cost-effective means for ensuring that SSMPs are developed and implemented appropriately;

- It is the Water Board’s desire to gather information on the causes and sources of SSOs to determine the full extent of SSOs and consequent public health and/or environmental impacts occurring in the State;

- Both uniform SSO reporting and a centralized statewide electronic database is needed to collect information for the Water Board to effectively analyze SSOs;

- Information regarding SSOs must be provided to Regional Boards and other regulatory agencies in a timely manner and be made available to the public in a complete, concise, and timely fashion;

- Several of the Regional Water Boards are engaged in initiatives to address SSOs, with some Water Boards issuing waste discharge requirements or NPDES Permits to sanitary sewer system owners/operators within their jurisdictions;

- Environmental groups, Regional Water Boards, and the U.S. Environmental Protection Agency (U.S. EPA) have taken a range of enforcement actions on SSOs;

- California Water Code Section 13263 provides that the State Water Resources Control Board may prescribe general waste discharge requirements for a category of discharges if the Water Board finds or determines that:

- The discharges are produced by the same or similar operations

- The discharges involve the same or similar types of waste;

- The discharges require the same or similar treatment standards;

- The discharges are more appropriately regulated under general discharge requirements than individual discharge requirements; and

- The Water Board, after any necessary hearing, may prescribe waste discharge requirements in accordance with this section

- The issuance of a single general waste discharge requirement to the Permittees will:

a)	Reduce the administrative burden of issuing individual waste discharge requirements to each Permittee; and

- Provide for a unified statewide approach for the reporting and database tracking of SSOs.

c)	Provide consistent and uniform standards of performance, operations, and maintenance of sanitary sewer systems.

d)	Provide statewide consistency in reporting as per California Water Code Section 13263 and Health and Safety Code Section 5410 et. seq.

e)	Facilitate consistent enforcement for violations.

- The Water Board has notified the Permittees and all known interested parties of the intent to prescribe waste discharge requirements to develop SSMPs and report all SSOs.

- The Water Board has, at a public meeting on November ?, 2005, heard and considered all comments pertaining to the terms and conditions of this Order.

IT IS HEREBY ORDERED, that the Permittees, in order to meet the provisions contained in Division 7 of the California Water Code and regulations adopted hereunder, shall comply with the following:

# DEFINITIONS

1. 	Sanitary Sewer Overflow - A sanitary sewer overflow (SSO) is any overflow, spill, release, discharge or diversion of untreated sewage from a sanitary sewer system. SSOs include:

(i)   Overflows or releases of wastewater that reach surface waters;

(ii)  Overflows or releases of wastewater that do not reach surface waters;

(iii) Sewage backups into buildings and on private property that are caused by blockages or flow conditions within the publicly owned portion of a sanitary sewer system shall be reported in accordance with the SSO Reporting System described in the Monitoring and Reporting Program No. ---.

Note:  Although discharges caused by a blockage or other malfunction of a building lateral that is privately owned is not necessarily considered an SSO, reporting these spills is encouraged when sewage is discharged off of private property into streets, storm drains, or waters of the State.  These spills may be reported by the Permittee, in accordance with the SSO Reporting System described in the Monitoring and Reporting Program No. ---, but are not the responsibility of the Permittee.

- Sanitary Sewer System – Any system of pipes, pump stations, sewer lines, etc., upstream of the headworks used to collect and convey sewage to a treatment plant. Temporary storage and conveyance facilities (such as vaults, temporary piping, construction trenches, wet wells, impoundments, tanks, etc.) are considered to be part of the sanitary sewer system, and discharges of sewage to these facilities are not sanitary sewer overflows.

- Permittee - Any person, city, or agency, public or private that is enrolled under Order No. 2005-, “Statewide General Waste Discharge Requirements for Sanitary Sewer Overflow Reduction Program.”

- SSO Reporting System – Online spill reporting system, hosted, controlled, and maintained by the SWRCB.  The web address for this site is: www.---.waterboards.ca.gov (web address to be determined).  This online database is maintained on a secure site and is controlled by unique usernames and passwords.

- Untreated Sewage – Any volume of sewage or wastewater (domestic, industrial, or mixed) upstream of a treatment facility.

# B.	PROVISIONS

- The Permittee must comply with all conditions of this Order.  Any noncompliance with this Order constitutes a violation of the California Water Code and is grounds for enforcement action.

- Nothing in this WDR shall (1) be interpreted or applied in a manner inconsistent with the Federal Clean Water Act, (supercede a more specific or more stringent state or federal requirement in an existing permit, regulation, or administrative/judicial order or Consent Decree, or (2) prohibit a Regional Board from issuing an NPDES permit, or including any NPDES requirement, authorized under the Clean Water Act.

- The Permittee shall take all feasible steps to eliminate SSOs consistent with California Water Code, and implementing applicable regulations, including Regional Water Board Basin Plan requirements.  In the event that an SSO does occur, the Permittee shall take all feasible steps to contain and mitigate the impacts of an SSO.

- The Permittee shall also take all feasible steps to prevent sewage from discharging from storm drains into flood control channels and open ditches by blocking the storm drainage system and by removing the sewage from the storm drains.

- All SSOs, must be reported in accordance with Section E of this WDR.

- In any enforcement action, Water Boards will consider the efforts of the Permittee to contain, control, and clean up sewage spills from its sanitary sewer system as part of its consideration of the factors required by Section 13327 of the California Water Code.  Water Boards will also give consideration to whether or not the following factors apply:

- The permittee can identify the cause or likely cause of the discharge event;

- There were no feasible alternatives to the discharge, such as retention of untreated wastewater, reduction of inflow and infiltration, use of adequate backup equipment, or an increase in the capacity of the system as necessary to contain the design storm event identified in the SSMP.  This provision is not satisfied if the agency does not undergo a periodic or continuing planning process to identify and correct problems.

- The discharge was exceptional, unintentional, temporary, and caused by factors beyond the reasonable control of the permittee;

- The discharge could not have been prevented by the exercise of reasonable control described in a certified SSMP and consistent with accepted and approved industry standards for:

- Proper management, operation and maintenance program;

- Adequate treatment facilities or sanitary sewer system facilities or components (e.g., adequately enlarging treatment or collection facilities to accommodate growth, I/I, etc…);

- Preventive maintenance (including cleaning and FOG control);

- Installation of adequate backup equipment; and

- Adequate inflow and infiltration prevention and control.

- Whether the spill was caused by a wet weather event that exceeded the design capacity of the collection system as documented in the SSMP

- The permittee took all reasonable steps to stop, and mitigate the impact of the discharge as soon as possible.

- Regulatory discretion – in any civil enforcement proceeding by a Water Board, a Water Board will consider the Permittee’s SSMP adequacy and level of implementation.  SSMP certification and audits will provide Water Boards with information associated with SSMP design, implementation, whether or not a Permittee is in compliance with industry standards, as well as, relative performance.

- Burden of proof - In any enforcement proceeding, the permittee has the burden of proof to establish that the criteria in this section have been met.

- The Regional Board will consider all SSMP certification documents, audit reports, and any other relevant material when considering an enforcement action, including, but not limited to, actions for violation of the Clean Water Act, Basin Plan prohibitions, or these waste discharge requirements.

- When a sanitary sewer overflow, the Permittee shall, to the extent necessary to maintain compliance with this Order, take all feasible steps and necessary remedial action to 1) control or limit the volume of sewage discharged, 2) terminate the sewage discharge as rapidly as possible, and 3) recover as much of the sewage discharged as possible for proper disposal, including any wash down water.  The Permittees shall implement all remedial actions to the extent they may be applicable to the discharge and consistent with an emergency response plan, including the following:

(i)	Interception and rerouting of sewage flows around the sewage line failure;

(ii)	Vacuum truck recovery of sanitary sewer overflows and wash down water;

(iii)	Cleanup debris at the overflow site.

- The Permittee shall properly, manage, operate, and maintain all parts of the sanitary sewer system owned and operated by the Permittee, with adequately trained staff and/or contractors possessing adequate knowledge, skills, and abilities.

- Each sanitary sewer system agency shall allocate adequate resources for the operation, maintenance, and repair of its sanitary sewer system, by establishing a proper rate structure, accounting mechanisms, and procedures to ensure an adequate and auditable measure of revenues and expenditures. These procedures must be in compliance with applicable existing laws and regulations and comply with generally acceptable accounting practices.

- The Permittee shall provide adequate capacity to convey base flows and peak flows, including wet weather related events to meet or exceed the design criteria as defined in the Permittee’s System Evaluation and Capacity Assurance Plan (as described in section D 13 (ix)), for all parts of the sanitary sewer system owned or operated by the Permittee.

- The Permittee shall develop and implement a written plan, a Sewer System Management Plan (SSMP), for compliance with these waste discharge requirements and make it available to any member of the public upon request in writing.  An SSMP must be adopted and approved by the elected board at a public meeting.

- The essential elements of the SSMP are specified below.  If the Permittee believes that any element of this section is not appropriate or applicable for their SSMP program, the program does not need to address it, but the SSMP must explain why that element is not applicable.

# Sewer System Management Plan (SSMP)

(i) 	Goals: The main goal of the SSMP is to prevent SSOs and to provide a plan and schedule for measures to be implemented to prevent SSOs.

(ii) 	Organization: The SSMP must identify:

(a) Administrative and maintenance positions responsible for implementing measures in the SSMP program, including lines of authority by organization chart or similar document; and

(b) The chain of communication for reporting SSOs, from receipt of a complaint or other information, including the person responsible for reporting SSOs to the Regional Water Board, County Health Officer, County Environmental Health Agency, and/or State Office of Emergency Services (OES); reporting to the OES is required if the discharge is 1,000 gallons or larger.

(iii) Legal Authority: Each Permittee must demonstrate, through collection system use ordinances, service agreements, or other legally binding procedures, that it possesses the necessary legal authority to:

    - Prevent illicit discharges into its wastewater collection system;

    - Require that sewers and connections be properly designed and constructed;

    - Ensure access for maintenance, inspection, or repairs for portions of the lateral owned or maintained by the Public Agency;

    - Limit the discharge of fats, oils, and grease and other debris that may cause blockages, and

    - Enforce any violation of its sewer ordinances.

(iv) 	Operation and Maintenance Program. In order to provide an adequate and appropriate SSO reduction plan, the SSMP must address the elements listed below that are appropriate and applicable to the Permittee’s system and identify the person or position in the organization responsible for each element:

    - Provide adequate operation and maintenance of facilities and equipment;

    - Maintain an up-to-date map of the sanitary sewer system showing all gravity line segments and manholes, pumping facilities, pressure pipes and valves, and applicable stormwater conveyance facilities;

    - Maintain relevant information to establish and prioritize appropriate SSMP activities and identify and illustrate trends in overflows, such as frequency and volume;

    - Routine preventive operation and maintenance activities by staff and contractors; including a system for scheduling regular maintenance and cleaning of the sanitary sewer system with more frequent cleaning and maintenance targeted at known problem areas.  The Preventative Maintenance (PM) program should have a system of tracking work orders and assessing the success of the PM program.

    - Where design criteria does not exist or is deficient, establish a program to assess the current capacity of the sanitary sewer system owned by the Permittee or where the Permittee has operational control;

    - Identify and prioritize system deficiencies and implement short-term and long-term rehabilitation actions to address each deficiency.  This shall include an appropriate rehabilitation plan including schedules for the entire system.  As with the PM program, sewer rehabilitation and replacement is crucial for the prevention of spills. Among the provisions that should be specified in this section is the need to direct rehabilitation and replacement at sewer pipes which are at risk of collapse or prone to more frequent blockages due to pipe defects.  The program should also include regular visual and TV inspection of sewer pipes and a system for assessing and ranking the condition of sewer pipes.  Finally, the rehabilitation and replacement plan should include a capital improvement plan that properly manages and protects the infrastructure assets.

    - Provide training on a regular basis for staff in sanitary sewer system operations, maintenance, and monitoring and determine if contractors’ staffs are appropriately trained;

    - Provide equipment and replacement part inventories, including identification of critical replacement parts.

    - Establish an implementation plan and schedule for a public education outreach program that promotes proper disposal of grease and fats.

    - Develop a plan and a schedule for the disposal of grease and fats generated within the sanitary sewer system service area.  This may include a list of acceptable disposal facilities and/or additional facilities needed to adequately dispose of FOG generated within a sanitary sewer system service area.

(v) Design and Performance Provisions:

(a) Develop design and construction standards and specifications for the installation of new sewer systems, pump stations and other appurtenances; and for rehabilitation and repair of existing sewer systems; and

(b) Develop procedures and standards for inspecting and testing the installation of new sewers, pumps, and other appurtenances and for rehabilitation and repair projects.

(vi)	Overflow Emergency Response Plan - Each Permittee shall develop and implement an overflow emergency response plan that identifies measures to protect public health and the environment.  At a minimum, this plan should include the following:

(a) Ensure proper notification procedures so that the primary responders are informed of all SSOs in a timely manner (to the greatest extent possible);

(b) Ensure that all overflows (including those that do not discharge to waters of the State) are appropriately responded to, including ensuring that reports of overflows are dispatched in a timely fashion to appropriate personnel for investigation and appropriate response;

(c) Ensure prompt notification of health agencies and other potentially effected entities (e.g., water suppliers) of all SSOs that potentially affect public health or reach the waters of the State in accordance with the MRP.  Report all SSOs in accordance with the MRP and State Law.  The SSMP should identify the public health agency and other officials who will receive immediate notification;

(d) Ensure that appropriate staff and contractor personnel are aware of and follow the plan and are appropriately trained;

(e) Provide emergency operations, such as traffic and crowd control and other necessary emergency response;

(f)	Take all reasonable steps to contain sewage and prevent sewage discharges to surface waters and minimize or correct any adverse impact on the environment resulting from the SSOs, including such accelerated or additional monitoring as may be necessary to determine the nature and impact of the discharge; and

(g)	Develop and implement a plan to respond in a timely manner to spills and other emergencies.  The response plan shall include the time it should take to respond and the appropriate emergency equipment the Permittee should have access to in order for proper response.

- (vii) Fats, Oils, and Grease Control Program:  Prepare and implement a fats, oil, and grease (FOG) source control program to reduce the amount of these substances discharged to the sanitary sewer system.    This plan shall include the legal authority to prohibit discharges to the system and identify measures to prevent SSOs caused by fats, oils, and grease blockages of sewers.   The elements of an effective grease control program shall include requirements to install grease removal devices (such as traps or interceptors), design standards for the removal devices, maintenance requirements, BMP requirements, record keeping and reporting requirements.  An effective grease control program must also include authority to inspect grease producing facilities, enforcement authorities, and sufficient staff to inspect and enforce the grease ordinance.

(a)	The grease control program shall identify sections of the sewer system subject to grease blockages and establish a cleaning maintenance schedule for each section; and

(b) The program shall develop and implement source control measures, for all sources of grease and fats discharged to the sewer system, for each section identified in (a) above.

(viii) System Evaluation and Capacity Assurance Plan: Prepare and implement a capital improvement plan (CIP) that will provide hydraulic capacity of key sewer system elements for dry weather peak flow conditions, as well as the appropriate design storm or wet weather event. At a minimum, the plan must include:

(a)	Evaluation: Steps to evaluate those portions of the sanitary sewer system, which are experiencing or contributing to an SSO discharge caused by hydraulic deficiency.  The evaluation must provide estimates of peak flows (including flows from SSOs that escape from the system) associated with conditions similar to those causing overflow events, estimates of the capacity of key system components, hydraulic deficiencies (including components of the system with limiting capacity) and the major sources that contribute to the peak flows associated with overflow events;

(b)	Capacity Enhancement Measures: Establish a short- and long-term CIP to address identified hydraulic deficiencies including prioritization, alternatives analysis, and schedules.  The CIP may include increases in pipe size, I/I reduction programs, increases and redundancy in pumping capacity, and storage facilities.

(ix)	Monitoring, Measurement, and Program Modifications

(a) Monitor the implementation and, where appropriate, measure the effectiveness of each element of the SSMP; and

(b) Update program elements, as appropriate, based on monitoring or performance evaluations.

(x)	SSMP Program Audits - As part of the SSMP, the permittee shall conduct periodic internal audits, appropriate to the size of the system and the number of overflows.  At a minimum these audits should occur every two years and a report should be prepared and kept on file.  This audit should focus on evaluating the effectiveness of the SSMP and its compliance with this subsection, including its deficiencies and steps to correct them.

(xi)	Communications Program - The Permittee shall communicate on a regular basis with the public on the development, implementation, and performance of its SSMP. The communication system shall allow the public the ability to provide input to the Permittee as the program is developed and implemented.

The Permittee shall also create a plan of communication with Satellite systems that are tributary to the Permittee’s sanitary sewer collection system.

16.	The SSMP and the Permittee’s program to implement the SSMP must be certified to be in compliance with the requirements set forth above.  Initially, the Permittee shall certify that their SSMP is in compliance with this WDR by sending a letter signed by the Agency’s authorized representative.  Letters shall be sent to:

Office of Statewide Initiatives

State Water Resources Control Board

P.O. Box 100

Sacramento, CA 95812

The State Board recognizes a need to develop a California collection system standards organization that will further develop standard collection system industry practices, certify collection system agencies to be in compliance with these standards, and provide the State Water Board and other governmental entities with information regarding program effectiveness and statewide resource needs.  However, until a California collection system standards organization is developed and has procedures in place to develop collection system industry standards and certify agencies to those standards, collection system agency self certification will be required.

The SSMP must be updated every five (5) years, and must include any significant program changes.  Re-certification by the governing Board of the permittee is required when significant updates to the SSMP are made.

17.	The Permittee shall comply with these requirements according to the following schedule.  This time schedule does not supersede existing requirements or time schedules associated with other regulatory requirements.

| ## Task | Completion Date |  |  |  |
| --- | --- | --- | --- | --- |
|  | Population > 100,000 | Population between 100,000 and 10,000 | Population between 10,000 and 2,500 | Population < 2,500 |
| Monitor and Reporting Program | Effective upon WDR Adoption | Effective upon WDR Adoption | Effective upon WDR Adoption | Effective upon WDR Adoption |
| SSMP Development Plan and Schedule | 5/1/2006 | 1/1/2007 | 4/1/2007 | 7/1/2007 |
| Legal Authority | 6/1/2006 | 3/1/2007 | 9/1/2007 | 3/1/2008 |
| Organizational Structure | 6/1/2006 | 3/1/2007 | 9/1/2007 | 3/1/2008 |
| Overflow Emergency Response Program | 12/1/2006 | 1/1/2007 | 7/1/2007 | 1/1/2008 |
| Operation and Maintenance Program | 3/1/2007 | 1/1/2008 | 7/1/2008 | 1/1/2009 |
| Grease Control Program | 8/1/2007 | 12/1/2008 | 9/1/2009 | 6/1/2010 |
| System Evaluation and Capacity Assurance Plan | 12/1/2007 | 3/1/2009 | 12/1/2009 | 9/1/2010 |
| Sewer Rehabilitation Plan | 6/1/2008 | 9/1/2009 | 6/1/2010 | 3/1/2011 |
| Final SSMP, incorporating all of the SSMP requirements | 10/1/2008 | 10/1/2010 | 10/1/2011 | 10/1/2012 |

### Sewer System Management Plan Time Schedule

# C.	WDR AVAILABILITY

- 1.	A copy of this WDR shall be maintained at appropriate locations and shall be available to sanitary sewer system operating and maintenance personnel at all times.

# D.	ENTRY AND INSPECTION

- 1.	The Permittee shall allow the Regional Board, or an authorized representative, upon presentation of credentials and other documents as may be required by law, to:

- Enter upon the Permittee's premises where a regulated facility or activity is located or conducted, or where records are kept under the conditions of this Order;

- Have access to and copy, at reasonable times, any records that must be kept under the conditions of this Order;

- Inspect at reasonable times any facilities, equipment (including monitoring and control equipment), practices, or operations regulated or required under this Order; and

- Sample or monitor at reasonable times, for the purposes of assuring compliance with this Order or as otherwise authorized by the California Water Code, any substances or parameters at any location.

# E.	GENERAL MONITORING AND REPORTING REQUIREMENTS

  - The Permittee shall furnish to the State or Regional Water Board, within a reasonable time, any information which the State or Regional Water Board may request to determine whether cause exists for modifying, revoking and reissuing, or terminating this Order.  The Permittee shall also furnish to the Executive Officer, upon request, copies of records, which are required to be kept, by this Order.

  - Pursuant to California Water Code Section 13267(b), the Permittee shall comply with the attached Monitoring and Reporting Program No.??? and future revisions thereto, as specified by the Executive Director.  Monitoring results shall be reported at the intervals specified in Monitoring and Reporting Program No.???.  These reporting requirements are intended to replace other mandatory routine written reports associated with SSOs.

  - Any person who, without regard to intent or negligence, causes or permits any sewage or other waste to be discharged in or on any waters of the State, or discharged in or deposited where it is, or probably will be, discharged in or on any surface waters of the State, as soon as that person has knowledge of the discharge, shall immediately notify the local health officer and the Regional Board of the discharge.  Discharges of sewage to storm drains and drainage channels, whether man made or natural or concrete lined, shall be reported as required above.

All SSOs greater than 1,000 gallons discharged in or on any waters of the State, or discharged in or deposited where it is, or probably will be, discharged in or on any surface waters of the State shall also be reported to the Office of Emergency Services pursuant to California Water Code Section 13271.

# F.	CHANGE IN OWNERSHIP

- 1.	This Order is not transferable to any person, except after notice to the Executive Officer.  The Permittee shall submit this notice in writing at least 30 days in advance of any proposed transfer.  The notice must include a written agreement between the existing and new Permittee containing a specific date for the transfer of this Order's responsibility and coverage between the existing Permittee and the new Permittee.  This agreement shall include an acknowledgement that the existing Permittee is liable for violations up to the transfer date and that the new Permittee is liable from the transfer date on.

# G.	INCOMPLETE REPORTS

- 1.	Where the Permittee becomes aware that it failed to submit any relevant facts in any report required under this Order, it shall promptly submit such facts or information.

# H.	REPORT DECLARATION

- 1.	All applications, reports, or information shall be signed and certified as follows:

(i)	All reports required by this Order and other information required by the State or Regional Water Board shall be signed and certified by a person designated, for a municipality, state, federal or other public agency, by either a principal executive officer or ranking elected official, or by a duly authorized representative of that person, as described in paragraph (ii) of this provision.

(ii)	An individual is a duly authorized representative only if:

- (a)	The authorization is made in writing by a person described in paragraph (i) of this provision;

- (b) The authorization specifies either an individual or a position having responsibility for the overall operation of the regulated facility or activity; and

(c)  All Permittees must obtain SSO Database accounts and receive a “Username” and “Password” by registering through the California Integrated Water Quality System (CIWQS).  These accounts will allow controlled and secure entry into the SSO Database.  Additionally, within 30days of receiving an account and prior to recording spills into the SSO Database, all Permittees must complete the “Collection System Questionnaire”, which collects pertinent information regarding a Permittee’s collection system.  The “Collection System Questionnaire” must be updated at least every 12 months.

# I.	CIVIL MONETARY REMEDIES FOR DISCHARGE VIOLATIONS

- 1.	The California Water Code provides that any person who violates this Order is subject to civil monetary remedies.

- 2.	The California Water Code also provides that any person failing or refusing to furnish technical or monitoring program reports, as required under this Order, or falsifying any information provided in the technical or monitoring reports is also subject to civil monetary penalties.

# J.	SEVERABILITY

1.	The provisions of this Order are severable, and if any provision of this Order, or the application of any provision of this Order to any circumstance, is held invalid, the application of such provision to other circumstances, and the remainder of this Order, shall not be affected thereby.

- 2.	This order does not convey any property rights of any sort or any exclusive privileges.  The requirements prescribed herein do not authorize the commission of any act causing injury to persons or property, nor protect the Permittee from liability under federal, state or local laws, nor create a vested right for the Permittee to continue the waste discharge.

# CERTIFICATION

The undersigned Clerk to the Board does hereby certify that the foregoing is a full, true, and correct copy of a resolution duly and regularly adopted at a meeting of the State Water Resources Control Board held on November 2005.

Debbie Irvin

Clerk to the Board

Ordered by:___________________________

Dated: